### Openscape Business

### OPENSTAGE 30 TELEPHONE

Your Openstage telephone has a display to assist you.  If you like the option that is displayed, press the    key.  If you want a different choice, use the      scroll key to move to the next option.

Preprogrammed key on your Openstage 30 telephone:-

PROGRAM	      	Service menu.

MAILBOX               	Calls the voicemail to access your mailbox.

LOUD SPEAKER    	Handsfree operation.

VOLUME     and  	Changes the volume and handset settings.

RELEASE                   	Hangs the current call up / Releases the call.

NUMBER REDIAL  	Press Redial = the last external number dialed is automatically dialed.

Press Redial key twice = the second last external number called is automatically dialed.

Press Redial key three times = the third last external number called is automatically dialed.

CALL FORWARD     	Forwards your phone to another number.

HEADSET             	Operates headset when headset is connected.

MUTE 		      	Turns microphone on/off.

The 6 keys on the side of the screen are programmed as:-

(2) General Call    			Incoming call answer and indication buttons.

SPARE KEYS				Keys not programmed.

PARK A CALL  			Places calls on hold in a Park position.

10 locations from 0 – 9.

HOLD  				Places answered call on hold.

| FUNCTION | ###### DESCRIPTION |
| --- | --- |
| Phone Settings | To change the speaker volume of the handpiece. - Whilst in conversation, press + or – key near handpiece. To change the ringer volume. - With the handpiece in cradle press + button Screen: Telephone settings - Make sure “Ringer volume” is highlighted on the screen. - Press: ##### Press + or – key until desired volume is reached. - Press: to save. To set the ringer tone. - With the handpiece in cradle press + button. Screen: Telephone settings - Scroll down so that “Ringer tone is highlighted on the screen. - Press: - Press + or – key until desired tone is reached. - Press: to save. |

| ### FUNCTION | ### DESCRIPTION |
| --- | --- |
| ### Internal Call | ### Lift handset and dial the extension number you require, or use DSS (Direct Station Selection) key. |
| ### External Call | ### Lift handset dial 0 for outside line, then dial the number you require. |
| ### Answering Calls | ### If the phone is ringing, lift the receiver. |
| ### To Answer more than one call. | ### You will have to decide whether you would like to:- ### Transfer call you are attending to – refer below. ### Place call on hold – refer below. ### Place call in park – refer below. |
| ### (a) Transfer Call | - Make sure “Enquiry” is highlighted on the screen. - Press key. - Key in extension number, or use DSS key. - Press Release key. # If the extension you are transferring to is busy or not answering the call, you can return to the call. - Make sure “Return to held call” is highlighted on the screen. # Press |
| ### (b) Hold | To put 1st caller on hold. - Make sure “Enquiry” is highlighted on the screen. - Press Hold key, then press Release key. (Your 1st caller is on hold and 1st Line key will flash slowly). You are now free to accept second call. To get your 1st caller back. - Press slow flashing Line key. |
| ### (c) Call Park | # To place call in park position. - Make sure “Enquiry” is highlighted # Press Park A Call key. # Enter and note park location (0-9). If park position number you entered is in use, enter another location. # Press Release key. # To retrieve a call from park position. # Press Park A Call key. # Enter park location number (0-9). |
| Handsfree Operation | - If speaking to caller using handpiece and you wish to use handsfree, press and hold Loudspeaker key until you place handpiece in cradle. |
| ### FUNCTION | DESCRIPTION |
| ### Call Back | If the internal extension you are trying to reach is busy or does not answer, you can store an automatic callback. - Make sure “Set Callback” is highlighted on the screen. - Press  # To Cancel: # Key in #58 on keypad. # Screen: Callback : Ext no: # Make sure “Delete” is highlighted on the screen. # Press  |
| ### Programming DSS keys (For extensions or outside numbers.) | You can program your spare keys on the phone to be DSS keys for numbers you regularly call. - Enter Code *91 Screen: Press feature key - Press key you require to program Screen: Not programmed - Make sure “Change feature” is highlighted on the screen. - Press Screen: Select feature - Make sure “Destinations” is highlighted on the screen. - Press Screen: Select feature - Make sure “Repdial Key” is highlighted on the screen. - Press Screen: Tel. no: - Dial 0 for outside line then required number, or dial extension number. - Make sure “Save entry” is highlighted on the screen. - Press Screen: Saved - Make sure “End” is highlighted on the screen. # Press |
| ### FUNCTION | DESCRIPTION |
| ### Voice Calling Announcement/ ### Intercom | This function allows you to issue a loud speaker announcement to another internal extension. # Pick up handpiece. # Key in *80 on key pad. # Dial the extension number. # Speak the announcement. # The extension user hears a bleep tone followed by an announcement over the telephone loudspeaker. Extension user can reply but needs to make sure the Mute button is off (no light). |
| Do Not Disturb | Extension users can activate DND so that they can’t receive calls but can make calls. The main telephone should never be put into DND. To activate - *97. To cancel - #97. |
| Conference Calls | You are able to have a conference with several telephones at once. - Dial required internal extension, or 0 then number for an external number. Screen: Enquiry - Make sure “Enquiry” is highlighted on the screen. - Press - Dial 2nd required internal extension, or 0 then number for an external number. - When called party answers scroll down to: “Conference”.  - Press If another party is required. - Make sure –Conference is highlighted on the screen. - Press - Make sure “Add to party” is highlighted on the screen. - Press then dial 3rd required internal extension or 0 then number for an external number. - When called party answers, scroll down to: “Conference”.  - Press |
| ### FUNCTION | DESCRIPTION |
| ### Individual Speed Dialling - ### Stores 10 numbers ### *0 to *9 | - Enter Code: *92 Screen: Abb. Dialling no: - Make sure “Next” is highlighted on the screen. - Press Screen: *0 - Scroll down so that “change entry” is highlighted on the screen. - Press Screen: *0: - Dial 0 – for outside line, then required number. - Make sure “Save entry” is highlighted on the screen. - Press Screen: *0: (number) Make sure “Next entry” is highlighted on the screen. - Press Screen: *1: - Scroll downso that “change entry” is highlighted on the screen. - Press - Dial 0 – for outside line, then required number. - Make sure “Save entry” is highlighted on the screen. - Press - Repeat above process until all required speed dial numbers have been entered. #### To change Individual Speed Dial numbers # Enter Code *92 Screen: Abb. Dialling no: - Make sure “Next” is highlighted on the screen. - Press Screen: *0: (number) - Make sure “Next” is highlighted on the screen if *0 is not the speed dial you wish to change and Press  - Repeat the last step until you reach the speed dial you wish to change. - Scroll downso that “Change entry” is highlighted on the screen. Press Screen: *0 - Dial 0 – for outside line, then required number. - Make sure “Save entry” is highlighted on the screen. - Press ###### To quit System Administration. Press Program/Service Key. ###### To dial a number using Individual Speed Dialling. Code *7 then speed dial code (*0 through to *9). |
| FUNCTION | DESCRIPTION |
| Group Speed Dialling – stores 1000 numbers from 000 to 999 | To program group speed dials it is necessary to start System Administration. This can only be done on the main phone. - Enter Code *95 Screen: System Administration user: - Press 1. - Press Screen: System Administration Ident/Password: - Press 1. - Press Screen: System admin. -User: 11 System speed dial 12 Time 13 Date - Make sure cursor is on 11 System speed dial. - Press Screen: 11 System speed dial 1 Speed dial number 2 Speed dial name Previous menu - Make sure cursor is on 1 Speed dial number. - Press Screen: 111 Abb. Speed dial number 000: + Next * Change - Scroll downto put cursor on * Change. - Press Screen: 111 Speed dial number Abb dial no: 000 F3 Delete Previous menu - Key in 0 for outside line then required number. - Make sure curser is on “Confirm”. - Press If entering more numbers:- Screen: 111 Abb. Speed dial number 000: (number) + Next * Change - Make sure curser is on “+ Next”. - Press - Scroll downto put cursor on * Change. - Press Screen: 111 Speed dial number Abb dial no: 001 F3 Delete Previous menu - Key in 0 for outside line then required number. - Make sure curser is on “Confirm”. - Press - Repeat above process until all required speed dial numbers have been entered. ###### To quit System Administration. Press Program/Service Key. ###### To dial a number using Group Speed Dialling. Code *7 then speed dial code (000 through to 999). |
| ### FUNCTION | DESCRIPTION |
| ### Call Forwarding/ Divert | You can call forward your phone to another telephone. - With the handpiece in the cradle press the Call Forward key. #### Screen: Diversion type: 1 = all calls 2 = only external calls 3 = only internal calls End - If you want to forward all calls make sure “1=all calls” is highlighted or scroll downso that your choice is highlighted on the screen. - Press Screen: all to: - Key in the extension number that calls are to be diverted to. Screen: all to: (number) - Make sure “Save entry” is highlighted on the screen. - Press To cancel Call Forwarding/Diversion - With the handpiece in the cradle press the Call Forward key. |
| Change Time | You can change the system time and date but only on the main telephone. - Enter Code *95 Screen: System Administration User: - Press 1. - Press  Screen: System Administration Indent Password: - Press 1. - Press  Screen: System admin – User: 11 System speed dial 12 Time 13 Date - Scroll downto put cursor on “12 Time”. - Press Screen: 12 Time Format: hhmm: - Make sure the cursor is on “* Change”. - Press Screen: 12 Time Format hhmm: - Key in correct time in 24 hour format. - Make sure the cursor is on “Confirm”. - Press To quit System Administration. Press Program/Service Key. |
| FUNCTION | DESCRIPTION |
| Call Transfer to ### Network (Note: This function needs to be programmed by a SATELCOTechnician) | You can transfer a call to an external number. Screen: Enquiry - Press - Press a DSS key if one has been programmed for the required number, or dial 0 for outside line then required number. - Wait for the person to answer. - Press Release key. The call has now been transferred. If the person you were ringing does not answer and you wish to return to the caller: - Make sure “Return to held call” is highlighted on the screen. - Press You are now connected to your original caller. |
| Group Pick Up (Note: Pickup groups can be programmed by a SATELCO Technician) | If your extension is in a Pick Up Group you can pickup calls for other extensions in your group. Screen: Call for: Pickup – Group - Press |
| Quick Reference Guide for Feature Codes Speed Number Dialling Call Forwarding Voicecall/Intercom Programming DSS keys Speed Number Programming Call Pick Up – Group Call Pick Up – Direct Clear unknown feature on extension Set Time and date | Individual *7 then library number *0 to *9 Group *7 then library number 000 to 999 *11 *80 then individual extension *91 Individual *92 Group *95 *57 *59 then ringing extension number Lift handset #0 *95 user 1 / password 1 (only on main phone) |